COAL INVENTORY Re-Order Point (ROP) Algorithm Barh · Dadri · Kudgi Power Plants Complete Guide: Concepts, Formulae & Python Implementation ¦ v3.0

✅ v3.0 Update: Min Level · Max Level · Reorder Level · Buffer Time · Stock_Status column added

# 1. What is RR Quantity and ROP?

## RR Quantity (Rake Receipt Quantity)

RR Quantity is the amount of coal received in one railway rake shipment. Each rake delivery is recorded with its date, plant destination, and quantity in metric tonnes (MT). In your dataset this is the rr_quantity column from the procurement file.

Typical rake size: 1 rake ≈ 3,800 – 4,200 MT (standard Indian coal logistics)

## ROP — Re-Order Point

The Re-Order Point is the stock level at which a new coal order must be placed immediately. The ROP is always fixed between the Minimum Level and the Maximum Level. When current stock falls to or below the ROP, the procurement team raises a new Rake Receipt order to ensure coal does not run out before the next delivery arrives.

The ROP algorithm monitors daily coal stock and automatically determines the minimum inventory level required to safely operate the power plant. The rate of consumption, number of days required to replenish stocks, and the maximum quantity of materials required on any day are all taken into consideration — along with a buffer time — when fixing the reordering level.

# 2. The Five Levels of Inventory Control

Version 3.0 introduces the full five-level inventory control framework. These levels form a hierarchy from critical minimum to overstocking ceiling:

| Level | Meaning | Formula |
| --- | --- | --- |
| Minimum Level | Safety floor — stock must NEVER fall below this | Reorder Level − (ADC × Avg LT) |
| ROP (Re-Order Point) | Trigger point: place a new order NOW | (ADC × Effective LT) + Safety Stock |
| Reorder Level | Upper trigger: stock is in the watch zone | Max Consumption × Max LT (incl. buffer) |
| Maximum Level | Overstocking ceiling: do not exceed this | Reorder Level + Reorder Qty − (Min Consumption × Min LT) |
| Safety Stock | Statistical buffer embedded in ROP | Z × σ × √(Effective Lead Time) |

# 3. Formulae

## 3.1 Average Daily Consumption (ADC)

ADC = Total Consumption ÷ Number of Days [Mean of Consumption (MT) column]

## 3.2 Lead Time & Buffer Time (NEW in v3.0)

Lead Time = average number of days between order placement and rake arrival at the plant.

Buffer Time = additional days added to lead time to absorb scheduling delays, transit variability, and operational risk.

Effective Lead Time = Lead Time + Buffer Time [Default: 7 + 2 = 9 days]

Two approaches are available:

- Fixed assumption: LEAD_TIME_DAYS = 7, BUFFER_DAYS = 2 (simple, use when lead time is consistent)

- Dynamic from data: avg(|rr_date − entry_dt|) per plant (more accurate; data shows Barh=1.5d, Dadri=2.7d, Kudgi=4.3d)

## 3.3 Safety Stock

Statistical approach (used in Python code):

Safety Stock = Z × σ × √(Effective Lead Time)

| Symbol | Meaning | Value |
| --- | --- | --- |
| Z | Service level factor | 1.645 for 95% ¦ 2.326 for 99% (critical plants) |
| σ | Std deviation of daily consumption | Computed from Consumption (MT) column |
| √LT | Square root of effective lead time | √9 = 3.0 for 9-day effective lead time |

## 3.4 Reorder Level

The Reorder Level is the upper trigger point — when stock enters this zone the procurement team should begin preparing an order.

Reorder Level = Max Consumption × Max Lead Time (Effective LT incl. buffer)

## 3.5 Minimum Level

The Minimum Level is the safety floor. Stock must never fall below this value under any operational scenario.

Minimum Level = Reorder Level − (Normal Consumption × Average Lead Time)

## 3.6 Maximum Level

The Maximum Level prevents overstocking. Ordering beyond this level ties up capital, consumes storage space, and creates obsolescence risk.

Maximum Level = Reorder Level + Reorder Qty − (Min Consumption × Min Lead Time)

## 3.7 Re-Order Point — Master Formula

The ROP is always fixed between the Minimum Level and the Maximum Level:

ROP = (ADC × Effective Lead Time) + Safety Stock

When current stock ≤ ROP → Place new order immediately

# 4. Worked Example — Real Data (Barh Plant)

| Step | Calculation | Value (from data) |
| --- | --- | --- |
| Avg Daily Consumption (ADC) | Mean of Consumption (MT) | 30,150 MT/day |
| Max Daily Consumption | Max of Consumption (MT) | 56,473 MT/day |
| Lead Time | Fixed assumption | 7 days |
| Buffer Time (NEW) | Scheduling buffer | 2 days |
| Effective Lead Time (NEW) | 7 + 2 | 9 days |
| Safety Stock | 1.645 × 7,729 × √9 | 38,141 MT |
| Reorder Level (NEW) | 56,473 × 9 | 508,257 MT |
| ROP — Trigger Level | (30,150 × 9) + 38,141 | 309,487 MT |
| Minimum Level (NEW) | 508,257 − (30,150 × 7) | 297,211 MT |
| Maximum Level (NEW) | 508,257 + 271,345 − (6,778 × 7) | 732,156 MT |

# 5. All Three Plants — ROP Summary (v3.0)

| Plant | ADC (MT/day) | Lead Time | Buffer Days | Safety Stock | Min Level | ROP (MT) | Max Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Barh | 30,150 | 7 | 2 | 38,141 | 297,211 | 309,487 | 732,156 |
| Dadri | 18,353 | 7 | 2 | 19,442 | 162,410 | 184,618 | 421,119 |
| Kudgi | 21,379 | 7 | 2 | 31,927 | 189,440 | 224,338 | 531,504 |

# 6. Stock Status Classification (NEW in v3.0)

Every row in the output now carries a Stock_Status column with five states:

| Status | Condition | Action Required |
| --- | --- | --- |
| CRITICAL | Stock ≤ Minimum Level | Emergency procurement — shutdown risk imminent |
| REORDER NOW | Stock ≤ ROP | Place order immediately — within lead time window |
| WATCH | ROP < Stock ≤ Reorder Level | Monitor closely, prepare purchase order |
| NORMAL | Min Level < Stock ≤ Max Level | No action needed |
| OVERSTOCK | Stock > Maximum Level | Pause or reduce incoming orders |

# 7. Days of Stock Remaining

Days_of_Stock = Current Stock (MT) ÷ ADC

Example: Stock = 440,226 MT | ADC = 30,150 MT/day → Days of Stock = 14.6 days (Barh, Apr 1 2023)

| Days of Stock | Status | Action |
| --- | --- | --- |
| > 14 days | Safe | No action needed |
| 7 – 14 days | Watch zone | Monitor closely, prepare order |
| ≤ 7 days (= Base Lead Time) | Critical — ROP triggered | Place order immediately |

# 8. Algorithm Logic (Step by Step)

Step 1: Load merged data (Date, Consumption (MT), Stock (MT), RR Quantity (MT), Allocated Cost, Cost per MT)

Step 2: Calculate ADC = mean(Consumption (MT))

Step 3: Set Lead Time = 7 days (fixed) OR compute avg(|rr_date - entry_dt|) per plant

Step 4: Set Buffer Time = 2 days; Effective LT = Lead Time + Buffer

Step 5: Calculate Safety Stock = Z x sigma x sqrt(Effective LT)

Step 6: Compute Reorder Level = Max Consumption x Effective LT

Step 7: Compute ROP = (ADC x Effective LT) + Safety Stock

Step 8: Compute Minimum Level = Reorder Level - (ADC x Lead Time)

Step 9: Compute Maximum Level = Reorder Level + Reorder Qty - (Min Consumption x Lead Time)

Step 10: Add Days_of_Stock, ROP_Level, Min_Level, Max_Level, Reorder_Level, Alert, Stock_Status to every row

Step 11: Export ROP summary table + per-plant sheets + alert CSV files

# 9. Parameters to Tune

| Parameter | Default | Change when... | Effect |
| --- | --- | --- | --- |
| LEAD_TIME_DAYS | 7 | Rail logistics change | Higher = larger ROP |
| BUFFER_DAYS | 2 | Supply reliability changes | Higher = more buffer in all levels |
| SERVICE_LEVEL | 0.95 (95%) | Risk tolerance changes | Higher = more safety stock |

Recommendation: For critical plants (Barh, Kudgi), consider SERVICE_LEVEL = 0.99 and BUFFER_DAYS = 3 to ensure 99% probability of no stock-out. This raises safety stock and all levels but eliminates shutdown risk.

# 10. Outputs

| Output File | Contents |
| --- | --- |
| rop_analysis_output_v3.xlsx | ROP_Summary sheet + Barh/Dadri/Kudgi sheets with ROP_Level, Min_Level, Max_Level, Reorder_Level, Alert, Days_of_Stock, Stock_Status columns |
| alerts_Barh.csv | Dates where Barh stock is at/below ROP — includes Stock_Status, Min_Level |
| alerts_Dadri.csv | Dates where Dadri stock is at/below ROP — includes Stock_Status, Min_Level |
| alerts_Kudgi.csv | Dates where Kudgi stock is at/below ROP — includes Stock_Status, Min_Level |
| Console output | Alert day count per plant, ROP / Min / Max values, full summary table |

# 11. Future: AI-Based Dynamic ROP

| Current (Statistical ROP) | AI-Based Dynamic ROP |
| --- | --- |
| ADC = historical mean | Forecasted demand from ML model |
| Buffer = fixed constant | Adaptive buffer from forecast uncertainty |
| Lead Time = fixed + buffer days | Predicted lead time from logistics data |
| ROP recalculated periodically | ROP updates daily with new forecast |

Next step: Replace ADC with a demand forecast and lead time with predicted transit time to get a fully dynamic ROP. The five-level framework (Min/ROP/Reorder/Max) remains the same — only the inputs change.

Column names must match exactly: Date · Plant · Consumption (MT) · Stock (MT) · RR Quantity (MT) · Allocated Cost · Cost per MT