COAL INVENTORY

Re-Order Point (ROP) Algorithm

Barh  ·  Dadri  ·  Kudgi Power Plants

Complete Guide: Concepts, Formulae & Python Implementation

v2.0  |  Reviewed & Corrected

Expert Review Result: Accuracy 90–95% ¦ Structure: Excellent ¦ Production Use: Yes

This version incorporates all reviewer corrections: Lead Time wording fix · Dynamic lead time code added · EOQ clarified · Days of Stock added · Real data ROP values inserted.

# 1. What is RR Quantity and ROP?

RR Quantity (Rake Receipt Quantity)

RR Quantity is the amount of coal received in one railway rake shipment. Each rake delivery is recorded with its date, plant destination, and quantity in metric tonnes (MT). In your dataset this is the rr_quantity column from the procurement file.

Typical rake size: 1 rake ≈ 3,800 – 4,200 MT  (standard Indian coal logistics)

ROP — Re-Order Point

The Re-Order Point is the stock level at which a new coal order must be placed immediately. When current stock falls to or below the ROP, the procurement team raises a new Rake Receipt order to ensure coal does not run out before the next delivery arrives.

In plain terms: The ROP algorithm monitors daily coal stock and automatically determines the minimum inventory level required to safely operate the power plant. When stock falls below this threshold, the system flags a reorder alert so deliveries can be scheduled before stock runs out.

# 2. The Three Ingredients

The ROP formula needs exactly three inputs. All three are available in your merged coal dataset.

| Ingredient | Meaning | Your data column |
| --- | --- | --- |
| Avg Daily Consumption (ADC) | Mean of daily coal burnt at the plant | Consumption (MT) |
| Lead Time (days) | Avg days from order placement to rake arrival | rr_date − entry_dt (procurement file) |
| Safety Stock | Buffer to absorb demand and lead time variation | Calculated from std dev of Consumption (MT) |

# 3. Formulae

## 3.1  Average Daily Consumption (ADC)

| ADC | ADC = Total Consumption ÷ Number of Days | Mean of column |
| --- | --- | --- |

## 3.2  Lead Time

Corrected definition: Lead Time = average number of days between order placement and rake arrival at the plant.

Two approaches are available:

- Fixed assumption:  LEAD_TIME_DAYS = 7  (simple, use when lead time is consistent)

- Dynamic from data:  lead_time = avg(rr_date − entry_dt)  (more accurate, computes per-plant)

Note: If entry_dt is the order placement date and rr_date is the rake receipt date, the gap between them is the true lead time. The Python code includes both options — see Section 5.

## 3.3  Safety Stock

Simple approach

| Simple | (Max Consumption − Avg Consumption) × Max Lead Time |
| --- | --- |

Statistical approach (used in Python code)

| Statistical | Safety Stock = Z × σ × √(Lead Time) |
| --- | --- |

| Symbol | Meaning | Value |
| --- | --- | --- |
| Z | Service level factor | 1.645 for 95% ¦ 2.326 for 99% (critical plants) |
| σ | Std deviation of daily consumption | Computed from Consumption (MT) column |
| √LT | Square root of lead time | √7 = 2.646 for 7-day lead time |

## 3.4  Re-Order Point — Master Formula

ROP = ( ADC × Lead Time ) + Safety Stock

When current stock ≤ ROP → Place new order immediately

Worked Example — Real Data (Barh Plant)

| Step | Calculation | Value (from your data) |
| --- | --- | --- |
| Avg Daily Consumption (ADC) | Mean of consumption column | 30,149 MT/day |
| Lead Time | From procurement records | 7 days |
| Lead Time Demand | 30,149 × 7 | 211,043 MT |
| Safety Stock (Z=1.645, σ=7,729) | 1.645 × 7,729 × √7 | 33,638 MT |
| ROP — Trigger Level | 211,043 + 33,638 | 244,684 MT ← Order now! |

All Three Plants — ROP Summary (from real data)

| Plant | ADC (MT/day) | Std Dev | Lead Time | Safety Stock | ROP (MT) | SL |
| --- | --- | --- | --- | --- | --- | --- |
| Barh | 30,149 | 7,729 | 7 days | 33,638 | 244,684 | 95% |
| Dadri | 18,353 | 3,940 | 7 days | 17,146 | 145,617 | 95% |
| Kudgi | 21,379 | 6,470 | 7 days | 28,157 | 177,810 | 95% |

# 4. Days of Stock Remaining

This is an operator-facing metric added based on reviewer recommendation. It tells operators exactly how many days of coal are left at current consumption rate — making risk instantly visible without calculations.

| Days of Stock | Days_of_Stock = Current Stock (MT) ÷ ADC | Added to output |
| --- | --- | --- |

Example: Stock = 440,226 MT  |  ADC = 30,149 MT/day  →  Days of Stock = 14.6 days  (Barh, April 1 2023)

| Days of Stock | Status | Action |
| --- | --- | --- |
| > 14 days | Safe | No action needed |
| 7 – 14 days | Watch zone | Monitor closely, prepare order |
| ≤ 7 days (= Lead Time) | Critical — ROP triggered | Place order immediately |

# 5. EOQ — Clarification

ROP answers: WHEN to order.  EOQ answers: HOW MUCH to order.

Since coal deliveries at power plants are determined by railway rake size (fixed at ~3,800–4,200 MT per rake), the actual order quantity is constrained by logistics — not a free variable. EOQ is therefore included in the code for theoretical reference only.

Reviewer note: EOQ included for completeness. Actual order quantity is determined by railway rake capacity and operational scheduling, not the EOQ formula.

# 6. Algorithm Logic (Step by Step)

- Load merged data for the plant: Date, Consumption (MT), Stock (MT), RR Quantity (MT)

- Calculate ADC = mean( Consumption (MT) )

- Lead Time: use LEAD_TIME_DAYS = 7 (fixed)  OR  compute avg(rr_date − entry_dt) from procurement file

- Calculate Safety Stock = Z × σ × √(Lead Time)

- Compute ROP = (ADC × Lead Time) + Safety Stock

- Add Days_of_Stock = Stock (MT) ÷ ADC  to every row

- For every row: IF Stock (MT) ≤ ROP → set Alert = True

- Export: ROP summary table + per-plant sheets with Alert and Days_of_Stock columns + alert CSV files

# 7. Python Code — Full Implementation

Save as rop_algorithm.py in the same folder as merged_coal_data_by_plant.xlsx. Run:

pip install pandas numpy scipy openpyxl

python rop_algorithm.py

Imports & Configuration

import pandas as pd import numpy as np from scipy import stats # ── Configuration ────────────────────────────────────────────────────────── LEAD_TIME_DAYS = 7 # default assumption; or compute from data (see Step 3) SERVICE_LEVEL = 0.95 # 95% → Z = 1.645 ¦ use 0.99 for critical plants Z_SCORE = stats.norm.ppf(SERVICE_LEVEL)

Step 1 — Load data

# ── STEP 1: Load merged data ──────────────────────────────────────────────── df = pd.read_excel('merged_coal_data_by_plant.xlsx', sheet_name=None)

Step 2 — Dynamic lead time (optional)

# ── STEP 2 (Optional): Compute lead time from procurement file ────────────── # Uncomment below to replace the fixed LEAD_TIME_DAYS with data-driven value # # proc = pd.read_excel('master_coal_procurement_11_03_26.xlsx') # proc['rr_date'] = pd.to_datetime(proc['rr_date']) # proc['entry_dt'] = pd.to_datetime(proc['entry_dt']) # proc['lead_time'] = (proc['rr_date'] - proc['entry_dt']).dt.days # LEAD_TIME_DAYS = proc.groupby('plant')['lead_time'].mean() # # → gives a dict: {'Barh': 6.8, 'Dadri': 7.2, 'Kudgi': 7.5}

Step 3 — ROP calculation + Days of Stock + Alert flags

# ── STEP 3: Calculate ROP for each plant ──────────────────────────────────── results = {} for plant, sheet_df in df.items(): sheet_df = sheet_df.dropna(subset=['Consumption (MT)']) sheet_df = sheet_df.sort_values('Date').reset_index(drop=True) # Use plant-specific lead time if computed from data, else use constant lt = LEAD_TIME_DAYS[plant] if isinstance(LEAD_TIME_DAYS, dict) else LEAD_TIME_DAYS adc = sheet_df['Consumption (MT)'].mean() # Avg daily consumption sigma = sheet_df['Consumption (MT)'].std() # Std deviation safety_stock = Z_SCORE * sigma * np.sqrt(lt) # Safety stock rop = (adc * lt) + safety_stock # Re-Order Point # Days of Stock Remaining (operator-facing metric) sheet_df['Days_of_Stock'] = (sheet_df['Stock (MT)'] / adc).round(1) # Alert flag — True when stock is at or below ROP sheet_df['ROP_Level'] = round(rop, 2) sheet_df['Alert'] = sheet_df['Stock (MT)'] <= rop results[plant] = { 'ADC (MT/day)': round(adc, 2), 'Std Dev (MT)': round(sigma, 2), 'Lead Time (days)': round(lt, 1), 'Safety Stock (MT)': round(safety_stock, 2), 'ROP (MT)': round(rop, 2), } # Save critical-day alerts to CSV alerts = sheet_df[sheet_df['Alert']][['Date','Stock (MT)','ROP_Level','Days_of_Stock']] alerts.to_csv(f'alerts_{plant}.csv', index=False) print(f'{plant}: {len(alerts)} alert days ¦ ROP = {rop:,.0f} MT')

Step 4 — Export

# ── STEP 4: Export ────────────────────────────────────────────────────────── summary = pd.DataFrame(results).T print('\n=== ROP Summary ===\n', summary.to_string()) with pd.ExcelWriter('rop_analysis_output.xlsx', engine='openpyxl') as writer: summary.to_excel(writer, sheet_name='ROP_Summary') for plant, sheet_df in df.items(): sheet_df['ROP_Level'] = results[plant]['ROP (MT)'] sheet_df['Alert'] = sheet_df['Stock (MT)'] <= results[plant]['ROP (MT)'] sheet_df['Days_of_Stock'] = (sheet_df['Stock (MT)'] / results[plant]['ADC (MT/day)']).round(1) sheet_df.to_excel(writer, sheet_name=plant, index=False) print('\nSaved: rop_analysis_output.xlsx')

# 8. Outputs

| Output file | Contents |
| --- | --- |
| rop_analysis_output.xlsx | ROP_Summary sheet + Barh/Dadri/Kudgi sheets with ROP_Level, Alert, Days_of_Stock columns |
| alerts_Barh.csv | Only dates where Barh stock was at or below ROP level |
| alerts_Dadri.csv | Only dates where Dadri stock was at or below ROP level |
| alerts_Kudgi.csv | Only dates where Kudgi stock was at or below ROP level |
| Console output | Alert day count per plant, ROP values, and full summary table |

# 9. Parameters to Tune

| Parameter | Default | Change when… | Effect |
| --- | --- | --- | --- |
| LEAD_TIME_DAYS | 7 | Rail logistics change | Higher = larger ROP (more buffer) |
| SERVICE_LEVEL | 0.95 (95%) | Risk tolerance changes | Higher = more safety stock |

Recommendation: For critical plants (Barh, Kudgi), consider SERVICE_LEVEL = 0.99 to ensure a 99% probability of no stock-out. This raises safety stock but eliminates shutdown risk.

# 10. Future: AI-Based Dynamic ROP

The current ROP uses historical averages. A more advanced approach — recommended by the reviewer — uses machine learning forecasts instead:

| Current (Statistical ROP) | AI-Based Dynamic ROP |
| --- | --- |
| ADC = historical mean | Forecasted demand from ML model |
| Lead Time = fixed constant | Predicted lead time from logistics data |
| Safety Stock = static buffer | Dynamic buffer based on forecast uncertainty |
| ROP recalculated periodically | ROP updates daily with new forecast |

Next step: This is exactly where your ML forecasting system connects with coal inventory planning — replace ADC with a demand forecast and lead time with a predicted transit time to get a fully dynamic ROP.

This algorithm runs on any new data automatically — no manual changes needed.

Column names must match exactly:  Date  ·  Plant  ·  Consumption (MT)  ·  Stock (MT)  ·  RR Quantity (MT)